22 nm 6T FinFET SRAM – RUN LOG REPORT

Baseline → Optimization → Measurement → Verification

Purpose. This run log records the design changes, model/file progression, measured results, engineering decisions, and verification status used to evaluate the 6T FinFET SRAM area-reduction candidate.

| Run | Model / File | Change / Parameter | Result | Decision | Evidence |
| --- | --- | --- | --- | --- | --- |
| R0 | 6T_SRAM_Baseline | Original 22 nm layout | 1368 × 584 nm; 798,912 nm² | Baseline established | KLayout baseline measurement |
| R1 | 6T_SRAM_optimized | Lateral geometric compaction | Width target: 1121.76 nm; height retained | Continue candidate evaluation | Optimized layout |
| R2 | 6T_SRAM_optimized | Selected lateral dimensions and horizontal spacing | Layer dimensions scaled to optimized values | Continue verification | Layer-by-layer audit |
| R3 | 6T_SRAM_optimized | Structural audit | 18 FIN; 36 GATES; 36 ACTIVE retained | Architecture preserved | Layer/count audit |
| R4 | 6T_SRAM_optimized | Final geometric measurement | 655,107.84 nm²; 143,804.16 nm² saved; 18.00% reduction | Target achieved; optimized selected | KLayout measurement |
| R5 | 6T_SRAM_optimized. analysis | SEMulator3D virtual fabrication | Process-integration verification | Final manufacturability gate | SEMulator3D outputs |

PARAMETER CHANGE RECORD

| Parameter / Layer | Baseline | Optimized | Reason for Change |
| --- | --- | --- | --- |
| Cell width | 1368 nm | 1121.76 nm | Reduce lateral footprint |
| Cell height | 584 nm | 584 nm | Preserve vertical dimension |
| FIN | 108 nm | 88.56 nm | Lateral compaction |
| NWELL | 128 nm | 104.96 nm | Lateral compaction |
| ACTIVE | 108 nm | 88.56 nm | Lateral compaction |
| GATE | 250 nm | 205 nm | Lateral compaction |
| GATECUT | 40 nm | 32.8 nm | Lateral compaction |
| LI / V0 | 48 / 30 nm | 39.36 / 24.6 nm | Interconnect/contact compaction |
| M1 / V1 / M2 | 36 / 36 / 36 nm | 29.52 / 29.52 / 29.52 nm | Metal/via compaction |

ENGINEERING DECISION & STATUS

The optimized candidate was selected because it achieves the required 15–20% area-reduction target with an 18.00% reduction while preserving the 584 nm cell height and core 6T structural counts. The remaining acceptance step is SEMulator3D virtual-fabrication and process-integration verification. No manufacturability PASS is claimed until that evidence is obtained.