6T FinFET SRAM Optimization
Measurement & Geometrical Evidence Report

Baseline vs Final 18% Optimized Layout

# 1. Measurement Objective

This report documents the dimensional and area measurements of the baseline and final optimized 6T FinFET SRAM layout. Measurements are extracted directly from the GDS geometry to provide reproducible evidence for the reported footprint reduction.

# 2. Measurement Source and Validity

| Item | Value |
| --- | --- |
| Measurement source | KLayout and SEMulator 3D |
| Input | Baseline and optimized GDS layout geometry |
| Measurement type | 2-D geometrical measurement |
| Baseline GDS | FinFET.gds |
| Optimized GDS | SRAM_6T_FINAL_SEMULATOR3D.gds |
| Database unit | 9.999999999999997e-07 |

Important evidence boundary: These are 2-D GDS geometrical measurements. SEMulator3D fabricated film thickness is not derived from GDS, and no fabricated thickness value is claimed here.

# 3. Overall Cell Dimensions and Area

| Metric | Baseline | Optimized | Reduction |
| --- | --- | --- | --- |
| Cell length / X | 1368.000 nm | 1121.760 nm | 18.00% |
| Cell width / Y | 584.000 nm | 584.000 nm | 0.00% |
| Bounding-box area | 798,912.000 nm² | 655,107.840 nm² | 18.00% |
| Bounding-box area | 0.798912000 µm² | 0.655107840 µm² | 18.00% |
| Area saved | — | 143,804.160 nm² | — |

Area reduction = (798,912.000 − 655,107.840) / 798,912.000 × 100 = 18.00%

# 4. Target Verification

| Parameter | Result |
| --- | --- |
| Challenge target | 15–20% area reduction |
| Measured area reduction | 18% |
| Target error | 0 percentage points |
| X scale factor | 0.82 |
| Y scale factor | 1 |
| Optimization direction | X-direction compression |
| Y footprint | Preserved |
| Verification status | PASS |

# 5. Layer-by-Layer Geometrical Measurements

| Layer | Shapes | Base X span nm | Base Y span nm | Base area µm² | Optimized X span nm | Optimized Y span nm | Optimized area µm² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NWELL | 18 | 1080.000 | 584.000 | 0.378432000 | 885.600 | 584.000 | 0.310314240 |
| FIN | 18 | 1188.000 | 544.000 | 0.352512000 | 974.160 | 544.000 | 0.289059840 |
| ACTIVE | 36 | 1324.000 | 544.000 | 0.637312000 | 1085.680 | 544.000 | 0.522595840 |
| GATEUNCUT | 36 | 1324.000 | 446.000 | 0.127104000 | 1085.680 | 446.000 | 0.104225280 |
| GATECUT | 36 | 1324.000 | 544.000 | 0.131648000 | 1085.680 | 544.000 | 0.107951360 |
| LI | 72 | 1236.000 | 540.000 | 0.112896000 | 1013.520 | 540.000 | 0.092574720 |
| V0 | 90 | 1332.000 | 552.000 | 0.096228000 | 1092.240 | 552.000 | 0.078906960 |
| M1 | 72 | 1332.000 | 552.000 | 0.159840000 | 1092.240 | 552.000 | 0.131068800 |
| V1 | 72 | 1332.000 | 552.000 | 0.062208000 | 1092.240 | 552.000 | 0.051010560 |
| M2 | 72 | 1332.000 | 552.000 | 0.342144000 | 1092.240 | 552.000 | 0.280558080 |
| M2_L1 | 36 | 1224.000 | 584.000 | 0.378432000 | 1003.680 | 584.000 | 0.310314240 |
| M2_L2 | 36 | 1368.000 | 584.000 | 0.420480000 | 1121.760 | 584.000 | 0.344793600 |

# 6. Critical-Dimension Proxy Measurements

| Layer | Min X nm | Max X nm | Min Y nm | Max Y nm | Geometry area µm² |
| --- | --- | --- | --- | --- | --- |
| FIN | 88.560 | 88.560 | 200.000 | 200.000 | 0.289059840 |
| ACTIVE | 88.560 | 100.040 | 148.000 | 200.000 | 0.522595840 |
| GATEUNCUT | 205.000 | 205.000 | 16.000 | 16.000 | 0.104225280 |
| GATECUT | 32.800 | 41.000 | 100.000 | 100.000 | 0.107951360 |
| LI | 39.360 | 131.200 | 24.000 | 24.000 | 0.092574720 |
| M1 | 29.520 | 91.020 | 36.000 | 96.000 | 0.131068800 |
| M2 | 29.520 | 29.520 | 102.000 | 208.000 | 0.280558080 |
| M2_L1 | 59.040 | 59.040 | 240.000 | 240.000 | 0.310314240 |
| M2_L2 | 59.040 | 59.040 | 240.000 | 240.000 | 0.344793600 |

# 7. Structural Verification

| Structure | Baseline | Optimized | Status |
| --- | --- | --- | --- |
| FIN | 18 shapes | 18 shapes | Preserved |
| ACTIVE | 36 shapes | 36 shapes | Preserved |
| M2 | 72 shapes | 72 shapes | Preserved |
| M2_L1 | 36 shapes | 36 shapes | Preserved |
| M2_L2 | 36 shapes | 36 shapes | Preserved |

M2, M2_L1 and M2_L2 are verified from the actual GDS layer/datatype mapping.

# 8. Baseline vs Optimized Final Result

| Metric | Baseline | Optimized | Change | Interpretation |
| --- | --- | --- | --- | --- |
| Cell X dimension | 1368.000 nm | 1121.760 nm | −246.240 nm | 18.00% reduction |
| Cell Y dimension | 584.000 nm | 584.000 nm | 0 nm | Preserved |
| Cell footprint | 798,912.000 nm² | 655,107.840 nm² | 143,804.160 nm² saved | 18.00% reduction |
| Optimization | Baseline | X-direction compression | Scale factor 0.82 | Lateral scaling |

# 9. Engineering Interpretation

• The optimization is predominantly lateral: X is compressed by a factor of 0.82 while Y is preserved.

• The measured footprint decreases from 798,912.000 nm² to 655,107.840 nm², giving an 18.00% reduction.

• The 18% result lies inside the specified 15–20% target window.

• Verified layer/structure counts are preserved for FIN, ACTIVE, M2, M2_L1 and M2_L2.

• KLayout measurements provide direct 2-D layout evidence; SEMulator3D outputs used separately for virtual fabrication and process-integration evidence.

# 10. Conclusion

The final optimized 6T FinFET SRAM layout demonstrates a measured 18.00% reduction in 2-D cell footprint, from 798,912.000 nm² to 655,107.840 nm². The reduction is achieved through X-direction compression while preserving the 584 nm Y dimension and verified structural layer counts. This report establishes the geometrical evidence for the area-scaling.